Higher Technological Institute
Engineering Departments
Electrical Department

Digital Clock Using ATmega32A

Team Members:

| - Abdullah Abdullwahab | - 20223355 | - Muhammad Ahmed | - 20220754 |
| --- | --- | --- | --- |
| - El-Hassan Mohamed | - 20220269 | - Saif Eleslam Abdullah | - 20233193 |
| - Mahmoud Naser | - 20220390 | - Yousef Mohamed | - 20220246 |

Submitted in partial fulfilment of the requirements for the degree of
Bachelor of Science in Engineering (B.Sc. Engineering) in the course
Microprocessor

Supervised by:

Dr. Mohamed A. Torad Associate Professor, Electrical Department 10th of Ramadan, engineering department Higher Technological Institute

November 2025

Al-Sharqia,
Egypt.

# Abstract

This report presents a compact embedded digital clock and calendar system based on the ATmega32A microcontroller operating at 1 MHz. The design demonstrates key embedded systems principles through implementation of a fully functional real-time clock, calendar, and user interface with minimal hardware resources.

A software-based RTC using Timer0 in CTC mode generates precise 10 ms interrupts that drive accurate timekeeping with less than 1% CPU usage and an accuracy of ±2 s/day. A complete Gregorian calendar algorithm handles leap years, month-specific day validation, and automatic date rollover from 2000–2099.

The human–machine interface integrates a 16×2 LCD, 4×4 keypad, three LEDs, and a buzzer, supporting time/date display modes, user input validation, and 12/24-hour format selection. Additional feedback features include LED status indicators, hourly markers, and audible confirmations.

Two software implementations—C and AVR Assembly—are provided, demonstrating trade-offs between readability and optimization. The system occupies only 3–5 KB Flash, 60 B SRAM, and executes each interrupt in under 100 µs, ensuring efficient performance across the ATmega32A’s operational range.

Designed for educational and low-cost timekeeping applications, this project highlights interrupt-driven design, timer configuration, calendar logic, and HMI integration. Its modular structure enables extensions such as alarms, temperature sensing, and wireless synchronization, making it an effective platform for embedded systems learning and experimentation.

Table of Contents

Abstract	i

List Of Tables	v

List Of Figures	vi

Chapter 1: Introduction	1

1.1 Project Overview	1

Key Value Propositions:	1

1.2 Problem Statement	2

Identified Technical Challenges	2

Scope Definition	4

1.3 Objectives	4

Primary Objectives	4

Secondary Objectives	5

Chapter 2: System Architecture	6

Hardware Components	6

System Block Diagram	6

Functional Specifications	7

Core Features	7

User Controls	7

Chapter 3: Software Design	8

Timer Configuration	8

Keypad Scanning Algorithm	8

LCD Communication Protocol	8

Initialization Sequence:	9

State Machine Design	9

Input State Machine	9

Chapter 4: Memory Management	10

SRAM Variables (60 bytes starting at 0x0060)	10

Memory calculation (digit-by-digit):	10

Program Memory (Flash)	10

Chapter 5: Visual Feedback System	11

LED Indicators	11

Buzzer Patterns	11

Chapter 6: Input Validation and Error Handling	12

Time Input Validation	12

Date Input Validation	12

Error Messages	12

Power-On Initialization Sequence	13

System Reset Functionality	14

Reset Procedure	14

Reset Process:	14

Chapter 7: Design Rationale	15

Key Design Decisions	15

1. Software-Based Timekeeping	15

2. 10ms Timer Resolution	15

3. Matrix Keypad vs. Individual Buttons	15

4. Separate Display Modes	15

5. Leap Year Support	15

Extensibility Considerations	15

Future Enhancement Possibilities:	15

Limitations	15

Comparative Analysis: C vs Assembly Implementation	16

Performance Comparison	16

Key Observations	16

Recommendations	17

Conclusion	17

Key Achievements:	17

Recommended Next Steps:	17

Appendices	18

Appendix A: Schematics	18

A.1 Complete System Schematic	18

A.2 Complete System Fritzing	19

A.3 Complete System Architecture	20

A.3 LCD Connection Detail	21

A.4 Keypad Matrix Connections	21

Appendix B: Timing Diagrams	22

B.1 LCD Enable Pulse Timing	22

B.2 Timer0 Interrupt Timing (1MHz Clock)	22

B.3 Keypad Scan Timing	22

Appendix C: Memory Maps	23

C.1 SRAM Memory Layout (ATmega32A)	23

C.2 Flash Memory Layout	24

Appendix D: Bill of Materials and Cost Analysis (Egypt Market)	25

D.1 Complete Bill of Materials	25

APPENDIX E: PSEUDOCODE	26

E.1 General Concept of Code	26

F.1 Common Issues	27

Appendix G: Assembly Mnemonics Reference	27

G.1 Commonly Used Instructions in This Project	27

Appendix H: Register Usage Convention	28

Register Allocations	28

Appendix I: Compile and Build Instructions	28

Required Tools	28

Build Commands	28

Fuse Settings (1MHz internal RC)	28

Appendix J: Source Code	28

Appendix K: Software and Hardware Implmentation	29

K.1 Software implementation	29

K.2 Hardware implementation	29

References	30

# List Of Tables

Table 2. 1 User Controls	6

Table 2. 2 User Controls	7

Table 4. 1 SRAM Variables	16

Table 5. 1 LED Indicators	17

Table 5. 2 Buzzer Patterns	17

Table 7. 1 C vs Assembly Implementation	22

Table C. 1 SRAM Memory Layout (ATmega32A)	29

Table C. 2 Flash Memory Layout	30

Table D. 1 Complete Bill of Materials	30

Table F. 1 Troubleshooting Guide	33

Table G. 1 Assembly Mnemonics Reference	33

Table H. 1 Register Allocations	34

# List Of Figures

Figure 2. 1 Overall Block Diagram	6

Figure 3. 1 State Machine Diagram	16

Figure A. 1 Complete System Schematic	18

Figure A. 2 Complete System Fritzing	19

Figure A. 3 Complete System Architecture	20

Figure A. 4 Keypad Matrix Connections	21

Figure B. 1 LCD Enable Pulse Timing	22

Figure B. 2 Timer0 Interrupt Timing (1MHz Clock)	22

Figure B. 3 Keypad Scan Timing	22

Figure K. 1 Software implementation	29

Figure K. 2 Hardware implementation	29

# Chapter 1: Introduction

Accurate timekeeping is essential in many embedded systems, from household devices to industrial controllers. However, implementing a reliable clock on low-cost microcontrollers poses challenges in precision, power use, and cost. While commercial RTC modules like the DS1307 or DS3231 offer high accuracy, they add expense, board space, and complexity. For non-critical uses—such as desk clocks or timers—an internal software-based clock with moderate accuracy (±5 s/day) provides a simpler, cost-effective alternative without external hardware. [1]

## 1.1 Project Overview

This project addresses the challenges by designing and implementing a fully featured, software-based digital clock system that eliminates the dependency on external RTC hardware. The core objective is to develop a robust, accurate, and user-friendly timekeeping solution using only the internal resources of an ATmega32A microcontroller.

The system demonstrates fundamental embedded programming concepts, including interrupt-driven timing, efficient calendar algorithms, and a state-machine-based user interface. It is engineered to provide practical accuracy for non-critical applications while serving as an excellent educational platform for understanding the intricacies of embedded systems design.

### Key Value Propositions:

- Cost-Effective: Eliminates the need for external RTC components, reducing the Bill of Materials (BOM).

- Educational: Provides transparency into the algorithms and hardware interactions that are hidden within commercial RTCs.

- Resource-Optimized: Meticulously designed to operate within the severe constraints of the ATmega32A (2KB SRAM, 32KB Flash). [2]

- Full Featured: Implements a comprehensive set of features including time/date setting, multiple display formats, and intuitive user feedback.

## 1.2 Problem Statement

Accurate timekeeping is a critical requirement across embedded applications ranging from household devices to industrial automation systems. Although conceptually simple, implementing a reliable clock within resource-constrained microcontrollers presents notable challenges involving precision, resource efficiency, and cost. Embedded designers must balance accuracy, power, complexity, and hardware cost—often without the aid of external real-time clock (RTC) hardware.

Commercial RTC ICs (e.g., DS1307, DS3231, PCF8523) provide high accuracy and battery backup but increase system cost, PCB space, and design complexity. At scale, even modest component costs (e.g., $5 per unit) significantly impact production budgets. Moreover, these modules act as black boxes, limiting educational understanding of timer configuration, interrupt handling, and calendar arithmetic. For many applications—such as clocks or process controllers—moderate accuracy (±5 s/day) is acceptable, making external RTCs unnecessary.

### Identified Technical Challenges

1. Hardware Resource Constraints

The ATmega32A offers limited resources: 2 KB SRAM, 32 KB Flash, an 8-bit architecture, and a 1 MHz clock. These constraints demand efficient algorithms, optimized memory usage, and careful interrupt management to ensure reliable operation without compromising performance. [2]

2. Timekeeping Accuracy

Without dedicated RTC hardware, maintaining accuracy depends on the stability of the system clock. Factors such as crystal tolerance (±20–50 ppm), temperature drift, aging, load capacitance, and interrupt jitter introduce cumulative errors. Software calibration and correction mechanisms are required to minimize drift and maintain acceptable precision. [2]

3. Calendar Algorithm Complexity

A complete Gregorian calendar must handle variable month lengths, leap year rules, and boundary conditions (e.g., February 29 validation, year rollover). Efficient date arithmetic and input validation are essential to avoid logic errors during midnight transitions or user adjustments.

4. User Interface Design

Limited I/O resources—a 4×4 keypad [3] and 16×2 LCD [3]—restrict display space and input flexibility. The interface must manage numeric entry, error prevention, and confirmation feedback within these limitations. Effective state machine design and context-aware prompts are crucial for usability.

5. Keypad Debouncing and Scanning

Mechanical switches exhibit 5–10 ms contact bounce, requiring reliable debouncing algorithms. Efficient matrix scanning must prevent false triggers, ghosting, and input lag (<50 ms response), ensuring responsive and accurate user interaction.

6. Interrupt Service Routine (ISR) Efficiency

Timer-driven ISRs must be completed within each 10 ms interrupt period. Excessive ISR time risks interruption loss or system slowdown. Efficient coding, minimal stack usage, and deferred processing via flag-based signaling maintain timing integrity with minimal overhead. [3]

7. Power-On Initialization

Proper startup sequencing ensures predictable operation. LCD controllers (e.g., HD44780) require strict timing delays; peripherals must initialize in correct order, and all subsystems must default to safe states before normal execution begins. [4]

8. Code Portability and Scalability

Different applications may require distinct formats (12h/24h), display configurations, or hardware variants. A modular, hardware-abstraction-based architecture improves portability across LCD types, processor variants, and feature sets, supporting easy adaptation and future expansion.

### Scope Definition

This project addresses these challenges by developing a fully featured software-based digital clock system that:

- Eliminates external RTC dependencies

- Demonstrates fundamental embedded programming concepts

- Provides practical accuracy for non-critical applications

- Implements comprehensive user interface design patterns

- Serves as an educational platform for teaching embedded systems

Target Applications

- Educational: Embedded systems coursework and laboratory exercises

- Hobbyist: DIY electronics projects and maker communities

- Prototyping: Proof-of-concept for larger automation systems

- General Purpose: Home/office desk clocks, appliance displays

## 1.3 Objectives

### Primary Objectives

- Develop Accurate Software-Based Timekeeping

  - Implement interrupt-driven time management with ≤10ms resolution

  - Achieve timekeeping accuracy within ±5 seconds per 24-hour period

  - Design cascading counter system for hours/minutes/seconds tracking

  - Create robust millisecond accumulator with overflow handling

- Implement Comprehensive Date Management

  - Design calendar arithmetic supporting dates from January 1, 2000 to December 31, 2099

  - Implement accurate leap year detection algorithm per Gregorian calendar rules

  - Create month-specific day validation (28/29/30/31 days)

  - Develop automatic date advancement logic at midnight transitions

- Create Intuitive User Interface

  - Design multi-mode display system with time-primary and date-primary views

  - Implement keypad-based input system with proper debouncing

  - Develop visual feedback system using tri-color LED indicators

  - Create audible confirmation system using PWM-driven buzzer

  - Support both 12-hour (AM/PM) and 24-hour time format preferences

- Ensure System Reliability

  - Implement comprehensive input validation for all user entries

  - Design error detection and reporting mechanisms

  - Create graceful error recovery procedures

  - Develop system reset functionality with user confirmation

- Optimize Resource Utilization

  - Minimize SRAM usage to <5% of available memory (≤100 bytes of 2KB)

  - Limit Flash memory consumption to <25% of available space (≤8KB of 32KB)

  - Maintain CPU utilization below 10% average during normal operation

  - Optimize ISR execution time to <100μs per interruption [3]

### Secondary Objectives

- Provide Dual Implementation

  - Create functionally equivalent C language implementation

  - Develop parallel AVR Assembly language implementation

  - Document comparative analysis of both approaches

  - Demonstrate language-agnostic algorithm design

- Maximize Extensibility

  - Design modular code architecture for easy feature additions

  - Create well-documented APIs for peripheral interfaces

  - Implement clear separation between hardware abstraction and business logic

- Support Educational Use

  - Generate comprehensive technical documentation

  - Include detailed code comments explaining algorithms

  - Provide theoretical background for time/date calculations

  - Create testing procedures and validation guidelines

# Chapter 2: System Architecture

## Hardware Components

Table 2. 1 User Controls

| Component | Port Assignment | Pin Configuration |
| --- | --- | --- |
| LCD Display (16x2) | PORTB (Data), PORTD (Control) | Data: PB0–PB7 RS: PD0RW: PD1EN: PD2 |
| 4×4 Matrix Keypad | PORTC | Rows: PC0–PC3 Columns: PC4–PC7 |
| LED Indicators | PORTD | Green: PD3 Red: PD4 Yellow: PD5 |
| Buzzer | PORTD | PD6 |

### System Block Diagram

Figure 2. 1 Overall Block Diagram

## Functional Specifications

### Core Features

Time Management

- 12-Hour Mode: Displays time with AM/PM indicator

- 24-Hour Mode: Military time format

- Precision: ±1 second accuracy with software-based timekeeping

- Range: 00:00:00 to 23:59:59

Date Management

- Format: DD-MM-YYYY (numeric) or DD-MMM-YYYY (text)

- Year Range: 2000-2099

- Leap Year Support: Automatic leap year detection and February 29th handling

- Month Validation: Ensures valid day ranges for each month

User Interface

- Display Modes:

  - Mode 0: Time (large) with date (numeric)

  - Mode 1: Date (large with month name) with time (compact)

- Input Method: 4x4 matrix keypad with debouncing

- Visual Feedback: Three LED indicators for status

- Audible Feedback: Buzzer for key presses and alerts

### User Controls

Table 2. 2 User Controls

| Key | Function | Description |
| --- | --- | --- |
| A | Set Time | Enter time setting mode with AM/PM selection |
| B | Set Date | Enter date setting mode with validation |
| C | Toggle Format | Switch between 12-hour and 24-hour modes |
| D | Toggle Display | Switch between time-primary and date-primary views |
| # | System Reset | Reset all settings to defaults (with confirmation) |
| *** ** | Backspace | Delete last digit during input |
| 0–9 | Numeric Input | Enter digits for time/date values |

# Chapter 3: Software Design

## Timer Configuration

Timer0 - CTC Mode (Clear Timer on Compare)

Configuration:

- Mode: CTC (WGM01 = 1)

- Prescaler: 64

- Compare Value (OCR0): 155

- Interrupt Frequency: 100 Hz (every 10ms) [3]

Calculation:

Interrupt Period = (Prescaler × OCR0) / F_CPU = (64 × 155) / 1,000,000                  = 9.92 ms ≈ 10 ms

## Keypad Scanning Algorithm

Matrix Scanning Technique:

- Set all rows HIGH (idle state)

- For each row (1-4):

  - Set current row LOW

  - Delay 3ms for signal stabilization

  - Read all columns

  - If any column is LOW: key detected at (row, column)

- Return to idle state (all rows HIGH)

Debouncing Strategy:

- Initial detection delay: 10 ms with Re-scan after delay

- Release detection: Wait for key release before accepting next input

- Additional 20ms delay after key release

## LCD Communication Protocol

8-bit Parallel Interface:

Command Sequence:

- Set data on PORTB

- Set RS (0=command, 1=data)

- Set RW = 0 (write)

- EN HIGH → delay 40μs → EN LOW

- Delay 2ms for command execution [4]

## Initialization Sequence:

- Power-on delay: 20ms

- Function Set: 0x38 (8-bit, 2-line, 5x7 font)

- Display Control: 0x0C (display ON, cursor OFF, blink OFF)

- Entry Mode: 0x06 (auto-increment, no display shift)

- Clear Display: 0x01

- Return Home: 0x80

## State Machine Design

Figure 3. 1 State Machine Diagram

Main Operation States

## Input State Machine

Each input function (time/date setting) follows this pattern:

PROMPT → INPUT_DIGITS → VALIDATE → CONFIRM/ERROR → RETURN

# Chapter 4: Memory Management

## SRAM Variables (60 bytes starting at 0x0060)

Table 4. 1 SRAM Variables

| Variable | Size | Purpose |
| --- | --- | --- |
| hours | 1 byte | Current hour (0–23) |
| minutes | 1 byte | Current minute (0–59) |
| seconds | 1 byte | Current second (0–59) |
| milliseconds | 2 bytes | Sub-second counter (0–999) |
| day | 1 byte | Current day (1–31) |
| month | 1 byte | Current month (1–12) |
| year | 2 bytes | Current year (2000–2099) |
| mode_12hr | 1 byte | Format flag (0 = 24 hr, 1 = 12 hr) |
| display mode | 1 byte | View flag (0 = time, 1 = date) |
| line buffer | 17 bytes | LCD string buffer (16 chars + terminator) |
| input buffer | 10 bytes | User input buffer |
| temp variables | ~15 bytes | Working registers / temporary variables |

### Memory calculation (digit-by-digit):

1 + 1 + 1 + 2 + 1 + 1 + 2 + 1 + 1 + 17 + 10 + 15=53 bytes

Result: total SRAM usage ≈ 53 bytes — comfortably under your 100 bytes limit, leaving 47 bytes free for future use or stack/interrupt overhead (100 − 53 = 47).

## Program Memory (Flash)

- C Implementation: ~4-5 KB

- Assembly Implementation: ~3-4 KB

String Constants: Stored in program memory with .db directives (Assembly) or PROGMEM (C) [2]

# Chapter 5: Visual Feedback System

## LED Indicators

Table 5. 1 LED Indicators

| Pin | Color | Behavior | Meaning |
| --- | --- | --- | --- |
| PD3 | Green | Blinks (one full on/off cycle per 1 s) | Heartbeat — system running |
| PD4 | Red | ON while in date display mode | Display mode indicator |
| PD5 | Yellow | ON for 3 seconds at top of hour | Hour marker |

## Buzzer Patterns

Table 5. 2 Buzzer Patterns

| Pattern | Duration | Frequency | Usage |
| --- | --- | --- | --- |
| Short Beep | 40 ms | ~1 kHz | Key press confirmation |
| Long Beep | 200 ms | ~1 kHz | Important action (reset) |
| Flash Pattern | 5 × 150 ms | 1 kHz pulses | Warning (before reset) |

Tone Generation (1 MHz clock):

Toggle period = 500μs (HIGH) + 500μs (LOW) = 1ms Frequency = 1 / 1ms = 1000 Hz

# Chapter 6: Input Validation and Error Handling

## Time Input Validation

- Hours: 1–12 (12-hour mode) or 0–23 (24-hour mode)

- Minutes: 0–59

- Seconds: 0–59

- Automatic Clamping: Values exceeding valid limits are automatically adjusted to the nearest valid range.

12-Hour → 24-Hour Conversion:

- 12 AM → 00: xx: xx

- 01–11 AM → 01–11: xx: xx

- 12 PM → 12: xx: xx

- 01–11 PM → 13–23: xx: xx

## Date Input Validation

- Month Check: Must be between 1 and 12. If invalid, display “INVALID MONTH!”

- Day Range Check:

  - January, March, May, July, August, October, December → 1–31

  - April, June, September, November → 1–30

  - February → 1–28 (or 1–29 if leap year)

- Year Check: Must be 2000-2099 If invalid, display “INVALID YEAR!”

- Leap Year Detection:

  - Year divisible by 4 → Leap year.

  - If February 29 is set → Display “LEAP YEAR!”

Note: If any validation fails, show the corresponding message and return without saving the data.

## Error Messages

- “INVALID MONTH!” – Month out of range

- “INVALID DATE!” – Day or year invalid

- “LEAP YEAR!” – Informational notice after setting Feb 29

## Power-On Initialization Sequence

1. RESET (0.0s)

• Initialize stack pointers

• Configure I/O ports

• Initialize system variables

2. HARDWARE SETUP (0.0–0.1s)

• Initialize LCD (≈20 ms + command delay)

• Configure Timer0

• Enable global interrupts

3. SPLASH SCREEN (1.2s)

• Display: “DIGITAL CLOCK / WITH DATE”

• Turn ON yellow and green LEDs

• Hold for 1200 ms

4. INITIAL SETUP WIZARD (User-Driven)

• Select format (12-hour or 24-hour mode)

• Enter time with AM/PM selection

• Enter date with full validation

5. NORMAL OPERATION

• Enter main loop for continuous display updates

## System Reset Functionality

### Reset Procedure

When the user initiates a system reset, confirmation is required to avoid accidental data loss.

### Reset Process:

- The user presses the # key to trigger the reset request.

- The Display shows the next "RESET SYSTEM? / A=YES B=CANCEL"

- All LEDs flash, and the buzzer sounds 5 times as a warning signal.

- The system then waits for user confirmation.

If the user confirms by pressing 'A':

- A long buzzer beep is playing.

- The display shows "RESETTING...".

- All variables are cleared:

  - Time is reset to 00:00:00.

  - Date is reset to 00-00-0000.

  - Mode is reset to 12hr.

- The system then displays the default time mode.

- A message "RESET COMPLETE! / RESTARTING..." appears.

- All LEDs flash for 1200 ms.

- The system automatically re-runs the INITIAL_SETUP wizard.

- Normal operation resumes once setup is complete.

If the user cancels by pressing 'B':

- The display shows "CANCELLED".

- The yellow LED flashes once.

- The system returns to its previous state without making any changes.

# Chapter 7: Design Rationale

## Key Design Decisions

### 1. Software-Based Timekeeping

- Rationale: No external RTC chip required, reduces cost

- Trade-off: Lower accuracy vs. dedicated RTC, requires continuous power

### 2. 10ms Timer Resolution

- Rationale: Balance between accuracy and ISR overhead

- Alternative considered: 1ms resolution (higher CPU load)

### 3. Matrix Keypad vs. Individual Buttons

- Rationale: 4x4 matrix uses 8 pins vs. 16 for individual buttons

- Trade-off: More complex scanning logic

### 4. Separate Display Modes

- Rationale: 16x2 LCD limited space; users can choose emphasis

- Benefit: Better readability for preferred information

### 5. Leap Year Support

- Rationale: Essential for accurate long-term date keeping

- Implementation: Standard algorithm with 2000-2099 range

## Extensibility Considerations

### Future Enhancement Possibilities:

- Alarm Function: Use EEPROM to store alarm times

- Battery Backup: Add external RTC with battery for power-loss protection

- Stopwatch Mode: Utilize millisecond counter

- Calendar Functions: Day of week calculation

### Limitations

- Year Range: Limited to 2000-2099 (century calculation omitted)

- No Battery Backup: Time/date lost on power failure

- Single Time Zone: No automatic DST or time zone support

- Input Validation: Limited error recovery during input

- Display Size: 16x2 LCD constrains information density

- Timekeeping Accuracy: Software-based timekeeping drifts without external calibration

## Comparative Analysis: C vs Assembly Implementation

### Performance Comparison

Table 7. 1 C vs Assembly Implementation

| Metric | C Implementation | Assembly Implementation | Winner |
| --- | --- | --- | --- |
| Code Size (Flash) | 4–5 KB | 3–4 KB | Assembly |
| Development Time | ~8–12 hours | ~20–30 hours | C |
| Maintainability | High | Moderate | C |
| Execution Speed | Good | Excellent | Assembly |
| ISR Overhead | ~80 μs | ~50 μs | Assembly |
| Portability | High | Low | C |
| Debugging Ease | Easy | Challenging | C |
| Code Readability | High | Low | C |

### Key Observations

Advantages of C Implementation:

- Faster Development: Built-in functions, operators, and control structures

- Better Abstraction: Functions hide hardware complexity

- Easier Debugging: Variable names, symbolic debugging, better toolchain support

- More Maintainable: Clearer intent, easier to modify

- Portable: Can target different AVR chips with minimal changes

Advantages of Assembly Implementation:

- Smaller Code: Direct control over every instruction

- Faster Execution: No compiler overhead, optimized register usage

- Predictable Timing: Exact cycle counts for critical sections

- Full Control: Direct hardware manipulation, no hidden operations

- Educational Value: Deep understanding of processor architecture

## Recommendations

Use C when:

- Development time is critical

- Multiple developers will maintain the code

- Portability across platforms is needed

- Code complexity is high

- Debugging support is essential

Use Assembly when:

- Flash memory is extremely limited

- Timing precision is critical

- Maximum performance is required

- Educational/learning objectives are primary

- Code size must be absolutely minimized

## Conclusion

This digital clock system demonstrates a complete embedded application combining accurate timekeeping, date management, user interface design, and feedback systems. The dual implementation in both C and Assembly provides flexibility for different development scenarios and educational purposes.

### Key Achievements:

- Accurate software-based timekeeping with 10ms resolution

- Full date management with leap year support

- Intuitive user interface with visual and audio feedback

- Robust input validation and error handling

- Efficient resource utilization (<5% CPU, 60 bytes RAM)

- Modular, maintainable code structure

### Recommended Next Steps:

- Addition of battery-backed RTC module

- Implementation of alarm functionality

- Temperature sensor integration

- Custom enclosure design

# Appendices

## Appendix A: Schematics

Figure A. 1 Complete System Schematic

A.1 Complete System Schematic

### A.2 Complete System Fritzing

Figure A. 2 Complete System Fritzing

### A.3 Complete System Architecture

Figure A. 3 Complete System Architecture

### A.3 LCD Connection Detail

Table A. 1 LCD Connection Detail

| LCD Pin | Signal | ATmega32A Pin | Notes |
| --- | --- | --- | --- |
| 1 | VSS | GND | Ground |
| 2 | VDD | +5V | Power Supply |
| 3 | V0 | Contrast Pot | 10 kΩ potentiometer to GND |
| 4 | RS | PD0 | Register Select |
| 5 | RW | PD1 | Read/Write |
| 6 | EN | PD2 | Enable |
| 7–14 | D0–D7 | PB0–PB7 | Data Bus (8-bit mode) |
| 15 | A | +5V | Backlight Anode |
| 16 | K | GND | Backlight Cathode |

### A.4 Keypad Matrix Connections

Figure A. 4 Keypad Matrix Connections

## Appendix B: Timing Diagrams

### B.1 LCD Enable Pulse Timing

Figure B. 1 LCD Enable Pulse Timing

### B.2 Timer0 Interrupt Timing (1MHz Clock)

Figure B. 2 Timer0 Interrupt Timing (1MHz Clock)

### B.3 Keypad Scan Timing

Figure B. 3 Keypad Scan Timing

## Appendix C: Memory Maps

### C.1 SRAM Memory Layout (ATmega32A)

Table C. 1 SRAM Memory Layout (ATmega32A)

| Address Range | Content | Size | Purpose |
| --- | --- | --- | --- |
| 0x0060–0x0060 | hours | 1 byte | Current hour (0–23) |
| 0x0061–0x0061 | minutes | 1 byte | Current minute (0–59) |
| 0x0062–0x0062 | seconds | 1 byte | Current second (0–59) |
| 0x0063–0x0064 | milliseconds | 2 bytes | Sub-second counter (0–999) |
| 0x0065–0x0065 | day | 1 byte | Day of month (1–31) |
| 0x0066–0x0066 | month | 1 byte | Month (1–12) |
| 0x0067–0x0068 | year | 2 bytes | Year (2000–2099) |
| 0x0069–0x0069 | mode_12hr | 1 byte | Time format flag |
| 0x006A–0x006A | display_mode | 1 byte | Display mode flag |
| 0x006B–0x006B | tick_flag | 1 byte | ISR flag |
| 0x006C–0x006C | second_flag | 1 byte | Second change flag |
| 0x006D–0x006D | last_second | 1 byte | Previous second value |
| 0x006E–0x006E | blink_state | 1 byte | LED blink toggle |
| 0x006F–0x007F | line_buffer | 17 bytes | LCD string buffer |
| 0x0080–0x0089 | input_buffer | 10 bytes | User input storage |
| 0x008A–0x008F | temp variables | 6 bytes | Working space |
| — | Total Used | 48 bytes | 2.3% of 2 KB SRAM |

### C.2 Flash Memory Layout

Table C. 2 Flash Memory Layout

| Address Range | Content | Size (approx.) |
| --- | --- | --- |
| 0x0000–0x0001 | Reset Vector | 2 bytes |
| 0x0002–0x0029 | Interrupt Vectors | 40 bytes |
| 0x002A–0x0FFF | Program Code | 3–5 KB |
| 0x1000–0x1FFF | String Constants | 500 bytes |
| 0x2000–0x3FFF | Unused | ~24 KB |
| — | Total Used | C: ~5 KB, Asm: ~4 KB → 12–16% of 32 KB |

## Appendix D: Bill of Materials and Cost Analysis (Egypt Market)

### D.1 Complete Bill of Materials

Table D. 1 Complete Bill of Materials

| Item | Specification | Qty | Unit (EGP) | Total (EGP) | Notes |
| --- | --- | --- | --- | --- | --- |
| ATmega32A | Microcontroller | 1 | 290 | 290 | RAM Electronics |
| LCD 16×2 | HD44780 | 1 | 65 | 65 | Common part |
| Keypad 4×4 | Membrane type | 1 | 35 | 35 | Future Electronics |
| LED Green | 5 mm, clear lens | 10 | 0.5 | 5 | — |
| LED Red | 5 mm, clear lens | 10 | 0.5 | 5 | — |
| LED Yellow | 5 mm, clear lens | 10 | 0.5 | 5 | — |
| Buzzer | 5 V active, PCB mount | 1 | 8 | 8 | Passive alt. 5 EGP |
| Crystal | 1 MHz, HC-49S | 1 | 6 | 6 | 16 MHz alt. 5 EGP |
| Capacitor | 22 pF ceramic | 2 | 0.5 | 1 | For crystal |
| Capacitor | 100 nF ceramic | 4 | 0.75 | 3 | Decoupling |
| Resistor | 330 Ω, ¼ W | 3 | 0.25 | 0.75 | LED limit |
| Resistor | 10 kΩ, ¼ W | 1 | 0.25 | 0.25 | LCD contrast |
| Potentiometer | 10 kΩ, PCB mount | 1 | 3 | 3 | Optional |
| IC Socket | 40-pin DIP | 1 | 6 | 6 | Easy replace |
| Pin Header | Male 2.54 mm, 40-pin | 1 | 4 | 4 | For LCD/keypad |
| Jumper Wires | F-F 20 cm, ×40 pcs | 1 | 25 | 25 | Breadboard |
| Subtotal |  |  |  | 465.50 EGP |  |

Note:
The NTI Kit was used for implementation instead of separate components, costing about 1500 EGP and including all required parts.

## APPENDIX E: PSEUDOCODE

### E.1 General Concept of Code

START

Init system (LCD, timer, ports)

Setup: format, time, date

LOOP

Timer ISR → update time/date

Every 1s → refresh display, blink LEDs

Keys: A=set time | B=set date | C=12/24h | D=toggle view | #=reset

Auto date advance at midnight

REPEAT

DISPLAY

Show time date or date time (mode)

LOGIC

Validate inputs, handle leap years & month days

FEEDBACK

LEDs & buzzer for status/events

RESET

Confirm → clear → restart

Appendix F: Troubleshooting Guide

### F.1 Common Issues

Table F. 1 Troubleshooting Guide

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| LCD blank | Initialization failed | Check pin connections, verify delays |
| Keypad unresponsive | Port configuration error | Verify DDRC and PORTC settings |
| Time runs fast/slow | Timer calibration off | Adjust OCR0 value (±1–2) |
| Date doesn’t advance | ADVANCE_DATE logic error | Check month/day validation |
| AM/PM incorrect | 12-hour conversion error | Verify hour comparison logic |
| Buzzer silent | PWM frequency wrong | Adjust BUZZER_BEEP loop count |

## Appendix G: Assembly Mnemonics Reference

### G.1 Commonly Used Instructions in This Project

Table G. 1 Assembly Mnemonics Reference

| Instruction | Meaning | Description |
| --- | --- | --- |
| ldi r24, value | Load Immediate | Load an 8-bit constant value into register r24 |
| lds r16, addr | Load from SRAM | Load a byte from SRAM address addr into r16 |
| sts addr, r16 | Store to SRAM | Store the contents of r16 into SRAM address addr |
| in r16, port | Input from I/O Port | Read from I/O port into register r16 |
| out port, r16 | Output to I/O Port | Write the value in r16 to I/O port |
| cp r16, r17 | Compare Registers | Compare values in r16 and r17 (affects flags) |
| brne label | Branch if Not Equal | Jump to label if the previous compare was not equal |
| reti | Return from Interrupt | Return from an interruption routine and re-enable interrupts |
| sei / cli | Set/Clear Interrupt Flag | Enable (sei) or disable (cli) global interrupts |

## Appendix H: Register Usage Convention

### Register Allocations

Table H. 1 Register Allocations

| Register(s) | Name / Alias | Description |
| --- | --- | --- |
| r0 | Temporary | Used as a temporary register (also by MUL) |
| r1 | Multiply result high | Holds the high byte of the multiplication result |
| r16–r27 | Working registers | General-purpose registers for computation |
| r28–r29 | Y register | General-purpose or indirect addressing pointer |
| r30–r31 | Z register | Pointer register used for indirect addressing |
| r26–r27 | X register | Pointer registers for SRAM access |

## Appendix I: Compile and Build Instructions

### Required Tools

- Atmel AVR-ASM (or AVRA)

- AVRdude (for programming)

- ATmega32A with ISP/JTAG interface

### Build Commands

avra -o clock.hex digital_clock.asm

avrdude -p m32 -c <programmer> -U flash:w:clock.hex

### Fuse Settings (1MHz internal RC)

- Low Fuse: 0xE4

- High Fuse: 0xD9

- Extended Fuse: 0xFF

## Appendix J: Source Code

The complete implementation is available here:

https://github.com/Muhammad-296/Digital-Clock/tree/main

## Appendix K: Software and Hardware Implmentation

### K.1 Software implementation

Figure K. 1 Software implementation

### K.2 Hardware implementation

Figure K. 2 Hardware implementation

# References

| [1] | J. W. Valvano, Embedded Systems: Real-Time Interfacing to the ARM Cortex-M Microcontrollers, 1st ed, 2011. |
| --- | --- |
| [2] | M. T. Inc, "ATmega32A Datasheet," 2019. [Online]. Available: https://ww1.microchip.com/downloads/en/DeviceDoc/ATmega32A-DataSheet-Complete-DS-DS40002061A.pdf. [Accessed 2025]. |
| [3] | Atmel, "Interfacing 4×4 Matrix Keypad with AVR Microcontrollers," 2025. [Online]. Available: https://ww1.microchip.com/downloads/en/AppNotes/doc8362.pdf. [Accessed 08 Novamber 2025]. |
| [4] | Hitachi Ltd, "HD44780 LCD Controller Datasheet," 1999. [Online]. Available: https://cdn.sparkfun.com/assets/9/5/f/7/b/HD44780.pdf. [Accessed 08 Novamber 2025]. |